ASBESTOS MANAGEMENT PLAN

William Cassidi C. E. Aided Primary School

Date Approved:			6th December 2023

Approved By:			PFS Committee

Next Review Date:		February 2025

Author/Reviewer:			SBC – Julie Cornelius

Version No.:			4

CONTENTS

1.0	ASBESTOS MANAGEMENT STATEMENT	3

2.0	SCHOOL MANAGEMENT FLOWCHART	5

3.0	ASBESTOS MANAGEMENT PLAN	6

3.1	Duty Holder	6

3.2	Responsible Persons	6

3.3	Damaged ACMs / Emergency Procedure	6

3.4	Asbestos Survey and Register	7

3.5	Asbestos Containing Materials	7

3.6	Staff Awareness	9

3.7	Monitoring and Inspection	9

3.8	Works and Visitor Protocols	10

3.8.1	Contractors	10

3.8.2	Emergency Services	10

3.9	Refurbishment or Demolition Works	10

3.10	Asbestos Works and Removals	11

Appendix 1: Asbestos Containing Material Inspection Record	12

Appendix 2: Contractor Review of Asbestos Register/Survey	14

Appendix 3: Asbestos Permit to Work Proforma	16

Permit to Work Issuing Instructions	16

Appendix 4: Staff Training Records	18

# 1.0	ASBESTOS MANAGEMENT STATEMENT

William Cassidi Aided Primary School has a local Asbestos Management Plan which can be found in the Schools Asbestos Documentation File.

This Asbestos Management Plan sets out the actions taken within the school premises to manage Asbestos Containing Materials (ACMs) in accordance with The Control of Asbestos Regulations (CAR), best practice guidance and Stockton Borough Council’s Asbestos Management Policy.

A copy of this plan and the premises asbestos survey and register, in addition to any other relevant information as detailed in the schools and local authority guidance, will be held in a central folder which can be found in the Schools Asbestos Documentation File located in the reprographics room.

This central folder will be made readily available to all those who need access to the asbestos documentation.

This plan has been developed in consultation with Stockton Borough Council, the schools leadership team and has been approved by the Governing Body.

To ensure school employees, pupils, contractors and visitors to the premises do not disturb ACMs and are safe from potential exposure, the following effective asbestos management procedures are in place:

- A designated person/s responsible for the management of asbestos on the schools premises (referred to as the Responsible Person); including the updating of existing records

- A system to ensure ACMs are identifiable through appropriate labelling and/ or colour coding

- Provision of asbestos awareness training to relevant school employees and third parties as deemed necessary (including the keeping of appropriate training records)

- To periodically inspect ACMs on a regular basis as specified within the asbestos register

- To periodically review this Asbestos Management Plan

- Provide access to the asbestos management central folder to contractors carrying out maintenance and/ or construction works (this includes IT contractors and Stockton Council’s Tradesmen) prior to the commencement of works

- Ensure that where deemed necessary, a refurbishment or demolition survey is undertaken when the school undertakes any construction works

- Seek advice and guidance from suitably qualified and experienced Competent Persons on any asbestos related work activities that are to be undertaken (this may include, but is not limited to: re-inspections, asbestos removal works, environmental cleans, encapsulation works & air-monitoring). In particular this will include the completion of a ‘Permit to Work’.

- Inform Stockton Borough Council of any instances of suspected exposure to ACMs so that the Council can provide professional assistance and guidance (refer emergency procedures)

# 2.0	SCHOOL MANAGEMENT FLOWCHART

Update Asbestos Survey & Register with Details

COPY OF ASBESTOS SURVEY/ REGISTER ISSUED TO SCHOOL

NOMINATE PERSON/S TO BE RESPONSIBLE FOR MANAGING

ASBESTOS CONTAINNG MATERIALS

(Provide them with learning and training)

IDENTIFY ASBESTOS CONTAINING MATERIALS THAT REQUIRE MANAGEMENT ACTION (Asbestos Register will identify)

LABEL

& / OR

COLOUR CODE

ACM’S

DAMAGED ACM’S FOUND: EMERGENCY PLAN TAKES EFFECT

ISSUE PAPERWORK FOR SCHOOL ACTIVITY

SCHOOL CARRIES OUT ADDITIONAL

INSPECTION OF ACM’S

AS PER MGMT PLAN RQMTS

INFORMS STAFF/ CONTRACTORS OF ACM’S AND CONTROL MEASURES

DEVELOP ASBESTOS MANAGEMENT PLAN

ASBESTOS RELATED WORKS UNDERTAKEN

ACTIVITY TAKES PLACE (Under Controlled Conditions Where Applicable)

ARRANGE FOR REFURB/ DEMOLITION SURVEY IF APPLICABLE

CONTRACTOR ISSUES PAPERWORK

ACCREDITED COMPANY CARRIES OUT SURVEY & FORMAL REINSPECTIONS

COUNCIL CORPORATE POLICY / SCHOOLS GUIDANCE

SCHOOL LOCAL POLICY

EXTERNAL COMPETENT PERSON INSPECTS ACMs ANNUALLY

PROTOCOLS FOR MGMT OF CONTRACTORS / SCHOOL ACTIVITIES

EMERGENCY PLAN ARRANGEMENTS IN PLACE

BUILDING WORKS

DETERMINE WHETHER ANY OF THE FOLLOWING APPLY:-

1. Review Asbestos Register/ Plan

2. Refurb/ Demolition Survey Required

3. Risk Assessment & Method Statement Required

4. Permit to Work Required

5. HSE Notification (Asb5)

BUILDING WORKS

DETERMINE WHETHER ANY OF THE FOLLOWING APPLY:-

1. Review Asbestos Register/ Plan

2. Activity Risk Assessment & Method Statement Required

3. Permit to Work Required

SCHOOL ISSUES PAPERWORK

DETERMINE WHETHER ANY OF THE FOLLOWING APPLY:-

1. Inform departmental safety manager/CFM

2. Review Asbestos Register/ Plan

2. Refurb/ Demolition Survey Required

3. Risk Assessment & Method Statement Required

4. Permit to Work Required

6. HSE Notification (Asb5)

DETERMINE WHETHER:-

1. HSE RIDDOR Notification Required

2. Incident Form Required

3. Notification to Staff, Pupils &/or Contractors Required

COUNCIL PROVIDES ADVICE & GUIDANCE

KEY: DARK BLUE TEXT      = SCHOOL ACTION

DARK GREEN TEXT    = COUNCIL ACTION

LIGHT BLUE TEXT       = CONTRACTOR ACTION

# 3.0	ASBESTOS MANAGEMENT PLAN

## 3.1	Duty Holder

The school has appointed the Headteacher as the legally designated Duty Holder (unless an alternative person has been identified and appointed by the Governing Body).

## 3.2	Responsible Persons

The following members of staff have been nominated to be responsible for managing asbestos on the schools premises.

- Julie Cornelius	-	Premise Nominated Person

Head Teacher

- Michelle Hardy	-	Premise Deputy Person

Caretaker

The members of staff detailed above have attended a recent asbestos awareness training course. Details of these records can be found in their training records.

## 3.3	Damaged ACMs  -  Emergency Procedure

Where asbestos containing materials (ACMs) are considered to have been accidental damaged or in the event of any concerns in relation to a risk posed by ACM’s the school will instigate the emergency procedure below.

- Secure the area affected ensuring no access is permitted (signage should be displayed and barriers erected where appropriate).

- Review impact on the schools operational procedures i.e. if a classroom is affected, alternative teaching arrangements would need to be implemented as the classroom cannot be used.

- Contact Stockton Councils Health & Safety Unit on 01642 526889 and the School Maintenance Officer (number detailed on the standard school contact sheet and also on the Asbestos Management Summary sheet) and notify them of the damage.

The Council’s Health & Safety Unit/Maintenance Officer will then provide professional advice and guidance as necessary. This may include, but is not limited to, inspecting the damage reported, arranging an air monitoring test and arranging and managing any associated remedial works required.

- Maintain controlled access to the area until such time as formal clearance has been confirmed.

- Maintain good communication with school staff and relevant other parties, providing updates as necessary to ensure the access arrangements are not breached.

3.4      Damaged ACM’s identified during routine inspections.

- The School Maintenance Officer will record the level of damage and then in accordance with the approved guidance make the necessary arrangement for remedial works to be completed.

- The School Maintenance Officer will arrange for the Asbestos Management Sheet to be updated to show that the ACM’s have been removed or are now suitably protected.

## 3.5	Asbestos Survey and Register

The Asbestos Survey provides accurate information on the location, amount and condition of ACMs. The information in the survey report will be used to form the asbestos register which is a key component of the management plan for the school.

The school will ensure that a copy of the asbestos survey will be available on the premises along with any other relevant information detailed in the Asbestos Management Plan.

This will be kept in the Schools Asbestos Documentation File

## 3.6	Asbestos Containing Materials

The areas of the school which have asbestos containing materials (ACMs) that require management will be noted from the asbestos register and detailed in the Asbestos Management Summary Sheet.

This will be kept in the Schools Asbestos Documentation File.

Where ACMs have been identified, they will be identified visually for all staff and contractors using the following:

- Asbestos containing materials in classrooms, corridors and other areas accessible to all staff and pupils will be identified by a ‘red dot’ label/ sticker (1.5’’ – 2’’), which will be referenced in the Asbestos Summary Sheet.

- Asbestos containing materials in other areas will be labelled using labels commensurate with legislative requirements, as per the samples in the table below.

| Examples of acceptable asbestos stickers/ labels to be used |  |
| --- | --- |
| The following examples of labels/ stickers are suitable for use within the school premises as part of the management control procedures outlined within this Asbestos Management Plan. These examples are not extensive and other appropriate stickers/ labels may be used. | Asbestos warning sticker – normal industry standard label used |
| Presumed asbestos sticker – used when similar materials have been proven to contain ACMs | Encapsulated asbestos sticker – used when ACMs have been encapsulated |
| Asbestos sticker – An alternative to the sticker highlighted above | Warning sticker – Can be used in communal areas where ACMs are present; may be used in place of other types specified above which may cause unnecessary concern |

## 3.6	Staff Awareness

The Duty Holder or the Nominated Person will inform all staff within the school with the following information:

- Types and location of ACMs (via the Asbestos Management Summary Sheet)

- The visual means of identifying ACM’s (labels/colour coding)

- How to avoid risks from asbestos (e.g. not disturbing)

- How to report concerns about ACMs (e.g. to the Duty Holder)

- How the Duty Holder/Nominated Person will inform the relevant Authorities of any accidental disturbance and what action the school needs to undertake.

- How periodic inspections are undertaken and the condition of ACMs is monitored and any remedial works actioned.

New staff are inducted into the health and safety procedures of the school which include Asbestos Awareness.

## 3.7	Monitoring and Inspection

Formal visual inspections of retained ACMs will be conducted on at least an annual basis, by the School Maintenance Officer who is qualified to BOHS P401 or P402 certificate level, or a suitably qualified alternative provider.

Any damaged or deteriorated materials found will be reported and remedial works actioned in accordance with the procedures detailed earlier in this plan at Section 3.4 (Damaged ACMs identified during routine inspections).

The date of the annual inspection programme is available via the School Maintenance Officer or Senior Surveyor (Asbestos Management) on 01642 528559 or 01642 526479.

## 3.8	Works and Visitor Protocols

## 3.8.1	Contractors

As part of the procedure to provide contractors with ‘Comprehensible Information’, every contractor attending the school to carry out any works will be provided with the Asbestos Summary Sheet, on arrival to the School, and prior to undertaking any work.

This will be provided by the person/s responsible for managing asbestos or other relevant member of staff within the school.

All contractors undertaking any work, having read and understood the sheet, will be required to sign that they have reviewed the Asbestos Summary Sheet. This will be undertaken via the signing of ‘Authorisation to work form’.

Works that involve the disturbance of any of the building fabric or building services can only be undertaken if the school have arranged for a Permit to Work to be completed in accordance with the  procedures detailed in Appendix 3 and has been duly authorised and implemented.

In the event of any concerns or disputes  relating to building works, or work to the building services, the school will contact the School Maintenance Officer or Senior Surveyor (Asbestos Management) on 01642 528559 or 01642 526479.

## 3.8.2	Emergency Services

Emergency Services personnel attending site must be given access to the Asbestos Summary Sheet on arrival.

## 3.9	Major Refurbishment or Demolition Works

Where the school commissions any construction works involving an upgrade, refurbishment or demolition work, the permit to work process will still be applied. Ultimately this will result in a refurbishment or demolition survey being undertaken in accordance with the requirements of the Health and Safety Executive guidance HSG264.

.

## 3.10	Asbestos Works and Removals

The school in liaison with the School Maintenance Officer, or the appointed Technical Officer for a specific project will ensure that any works undertaken involving ACMs will be carried out within the requirements of the Control of Asbestos Regulations 2012. Guidance from the Health and Safety Executive can be found at: http://www.hse.gov.uk/asbestos/index.htm.

Control measures will be detailed in the method statement provided by the contractor.

Where ACMs are to be removed or encapsulated, the Technical Officer will review the contractor’s method statement and authorise all the associated works.

The school asbestos register will be updated accordingly following completion of the asbestos related works.

Policy approved by the Governing Body: 9th February 2022

Date for Review: February 2024

# Appendix 1: Asbestos Containing Material Inspection Record

| School |  | Date of Inspection |  |
| --- | --- | --- | --- |
| Inspection By (signature) |  | Print Name |  |

Good Practice Guidance:

- Take photographs of ACMs for each inspection and store for future reference

- Compare previous photographs with current condition state to determine whether there has been any deterioration since the last inspection

- Replace missing/ damage asbestos labels/ stickers

- Where the condition is determined as either Medium or High, the emergency procedure needs to be instigated

Condition Key:

| Good: No visible damage; review at next inspection date | Low: A few scratches or surface marks but no signs of asbestos debris; review at next inspection date for further signs of deterioration |
| --- | --- |
| Medium: Significant breakage of materials or several small areas where material has been damaged; encapsulation works likely - follow the emergency procedure | High: Visible asbestos debris; environmental clean and encapsulation works likely – follow the emergency procedure |

| LOCATION OF ACM | ACM DESCRIPTION | (delete as applicable) CONDITION | INSPECTION COMMENTS |
| --- | --- | --- | --- |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |
|  |  | Good / Low Medium / High |  |

# Appendix 2: Contractor Review of Asbestos Register/Survey

This sheet should be signed by all those carrying out repair/maintenance work on the premises (including voluntary workers or staff) that will involve disturbing the fabric of the premises.

Persons signing this sheet are signing to say that they have seen the Asbestos Register and Management Plan and checked that whether there is any known or presumed asbestos in the area in which they are working.

Where an asbestos containing material is suspected, no work should be carried out until all relevant procedures have been carried out as detailed in this plan.

| Date | Company | Details of Work | Print Name | Signature |
| --- | --- | --- | --- | --- |

# Appendix 3: Asbestos Permit to Work Proforma

## Permit to Work Issuing Instructions

To be used when any work on/near asbestos containing materials is to take place

Section 4: Activity / Work NEAR asbestos containing materials

4.1 Has a method statement been prepared for the work?       Yes / No

The activity / works can commence once the above is confirmed but must stop immediately if any suspicious materials are discovered

ASBESTOS PERMIT TO WORK

Date/s for Works/ Activity:…………………………………………………………              Times:……………………………………

Section 1: Location and description of work

Section 2: Asbestos Register

2.1 Has the Asbestos Register been reviewed by the staff member/Contractor?   Yes / No

2.2 Will the work disturb any asbestos containing material?Yes / No

If the answer to 2.2 is YES – proceed to Section 3; if NO – proceed to Section 4

Section 3: Work ON asbestos containing materials

3.1 Contractors competency to work on ACM’s been confirmed?                       Yes / No

3.2 Worked planned in accordance with the requirements of The Control

of Asbestos Regulations 2012?       Yes / No

3.3 Southwark Council CFM been informed and given approval?                  Yes / No

Work can commence once the above are confirmed

Section 5: School Approval

Signature: ………………………………………………………………………………..Date:………………………………………………..

Print Name:…………………………………………………………………………….. (Responsible Person\

Section 6: Staff / Contractor Confirmation

I confirm that the activity / work will be undertaken in accordance with the requirements of this permit and school asbestos management plan.

Signature:………………………………………………………………………………..Date:………………………………………………..

Print Name:………………………………………………… Company Name:…………………………………………………………........

# Appendix 4: Staff Training Records

Staff’s training needs are subject to periodic review.

Periodic asbestos awareness training will be provided to those staff where the schools Training Needs Analysis (TNA) identifies the need. Other professional training will be undertaken as and when required/ determined by the TNA.

| Staff Name | Details of Training Attended | Date Training Attended | Next Training Date |
| --- | --- | --- | --- |

These training records should be reviewed as a minimum annually and should form part of the staff’s Personal Development Plan and Performance Review.